| PYTHON CODE | Annotations |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns | These lines of code are to import our packages that are necessary to analyze and use our data. |
| sns.set_style('whitegrid') sns.set_palette('deep') | These lines are used so that we have a clean visual for all of our plots. |
| nhts_data = pd.read_csv('NHTS.csv') ngsim_data = pd.read_csv('NGSIM.csv') | These lines import the data that we will be using. |
| print(sns.palettes.SEABORN_PALETTES.keys()) | This line is used to visualize Seaborn’s color palettes. |
| fuel_counts = nhts_data['fuel_type'].value_counts().dropna() print(fuel_counts) | These lines are used to count our values while removing the missing values. |
| plt.figure(figsize = (12,6)) xaxis = fuel_counts.plot(kind = 'bar', color = sns.color_palette('pastel', len(fuel_counts)), edgecolor = 'black') xaxis.bar_label(xaxis.containers[0]) | These lines of code are used to count the number of vehicles based on their fuel type. |
| plt.xlabel('Fuel Type', fontsize = 12) plt.xticks(rotation = 45, ha = 'right') plt.ylabel('Number of Vehicles', fontsize = 12) plt.title('Fuel Type vs. Number of Vehicles', fontsize = 15) | These lines create our bar plot and add customizations to it to make it easy to view and understand. |
| plt.savefig("FuelType vs. Number of Vehicles (Bar Chart).png", dpi = 300, bbox_inches = "tight") plt.show() | These two lines of code show the plot we created. |
| plt.figure(figsize = (12,6)) | This line of code tells PYTHON that you are creating a figure and determines the figure size. |
| urban_ages = nhts_data[nhts_data['household_location'] == 'Urban']['vehicle_age'] plt.hist(urban_ages, bins = 20, edgecolor = 'black', alpha = 0.7, color = 'lightseagreen', label = 'Urban') | These lines of code are used to filter and plot the Urban vehicle ages. |
| rural_ages = nhts_data[nhts_data['household_location'] == 'Rural']['vehicle_age'] plt.hist(rural_ages, bins = 20, edgecolor = 'black', alpha = 1, color = 'khaki', label = 'Rural') | These lines are used to filter and plot the Rural vehicle ages on the same figure. |
| plt.xlabel('Vehicle Age (years)') plt.ylabel('Count') plt.title('Vehicle Age Distribution Across Urban and Rural Areas') plt.legend(edgecolor = 'black', facecolor = 'white', framealpha = 1) | These lines add labels and customizations to the figure, making it easier to understand. |
| plt.savefig("Vehicle Age Distribution Across Urban and Rural Areas (Histogram).png", dpi = 300, bbox_inches = "tight") plt.show() | These lines of code are to display and visualize the created histogram. |
| plt.figure(figsize = (14,10)) | This line of code establishes a figure and its size. |
| sns.boxplot(data = nhts_data, y = 'vehicle_age', x = 'make', palette = 'crest') | This line of code specifies to create a boxplot and gives the different variables to plot. |
| plt.xticks(rotation = 90, fontsize = 12) plt.xlabel('Make', fontsize = 12) plt.ylabel('Vehicle age (years)', fontsize = 12) plt.title('Boxplot of Vehicle Age by Make', fontsize = 15) | These lines add the customization to the plot. |
| plt.tight_layout() plt.savefig("Vehicle Age by Make (Boxplot).png", dpi = 300, bbox_inches = "tight") plt.show() | These lines of code are used to show the created boxplot. |
| plt.figure(figsize = (12,5)) | This line of codes establishes a figure and its size. |
| sns.lineplot(x = ngsim_data['Time'], y = ngsim_data['follower_acc(m/s^2)'], color = 'teal') | This line of code tells PYTHON to create a Time-Series plot. |
| plt.xlabel('Time (seconds)', fontsize = 12) plt.ylabel('Acceleration (m/s^2)', fontsize = 12) plt.title('Follower Acceleration vs. Time', fontsize = 15) | These lines of code add the customizations to make the plot easy to understand. |
| plt.savefig("Follower Accerleration vs Time (Time 1).png", dpi = 300, bbox_inches = "tight") plt.show() | These two lines are used to show the plot. |
| plt.figure(figsize = (12,5)) | This line of code tells PYTHON to make a figure and determines the size. |
| sns.lineplot(x = ngsim_data['Time'], y = ngsim_data['follower_speed(m/s)'], color = 'palevioletred') | This line provides the type of plot and the data we are using. |
| plt.xlabel('Time (seconds)', fontsize = 12) plt.ylabel('Follower Speed (m/s)', fontsize = 12) plt.title('Follower Speed vs. Time', fontsize = 15) | These lines of code are used to add customizations to the plot for better visuals. |
| plt.savefig("Follower Speed vs. Time (Time 2).png", dpi = 300, bbox_inches = "tight") plt.show() | These two lines tell PYTHON to show the plot in a certain layout. |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): | This line of code starts the defining process for our acceleration estimation equation. |
| s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) s_star = max(s_star, s0) | These lines of code establish the desired minimum gap and make sure that it is not negative. |
| acceleration = a *(1 - (v/v0)**delta - (s_star/s)**2) return acceleration | These lines of code calculate the acceleration using the IDM equation. |
| trajectory_number = 1 | This line of code selects a trajectory pair to be used |
| data_subset = ngsim_data[ngsim_data['trajectory_number'] == trajectory_number].reset_index(drop=True) leader_position = data_subset['leader_position(m)'].values leader_speed = data_subset['leader_speed(m/s)'].values time_data = data_subset['Time'].values | These lines of code are used to extract the leader’s position and speed over time. |
| dt = time_data[1] - time_data[0] print(f" Time Step: {dt:.4f} s") | These lines determine what the time step is. |
| v0 = float(input("Enter desired velocity (m/s): ")) s0 = float(input("Enter minimum spacing (m): ")) T = float(input("Enter desired time headway (s): ")) a_param = float(input("Enter maximum acceleration (m/s^2): ")) b = float(input("Enter comfortable deceleration (m/s^2): ")) delta = float(input("Enter acceleration exponent: ")) | These lines of code establish the IDM parameters to be used in the calculations. |
| n_steps = len(time_data) sim_position = np.zeros(n_steps) sim_speed = np.zeros(n_steps) sim_acceleration = np.zeros(n_steps) | These lines of code determine the number of simulation steps taken, the simulated follower position with a vector, simulated follower speed with a vector, and simulated follower acceleration with a vector. |
| sim_position[0] = data_subset['follower_position(m)'].values[0] sim_speed[0] = data_subset['follower_speed(m/s)'].values[0] | These lines tell PYTHON to start at the real follower’s initial state. |
| for i in range(n_steps - 1): | This line is the beginning of running the IDM simulation. |
| gap = max(leader_position[i] - sim_position[i], 0.1) delta_v = sim_speed[i] - leader_speed[i] | These lines calculate the gap and speed difference. |
| sim_acceleration[i] = idm_acceleration(sim_speed[i], gap, delta_v, v0, s0, T, a_param, b, delta) | This line is to calculate the acceleration difference. |
| sim_speed[i+1] = sim_speed[i] + sim_acceleration[i] * dt sim_position[i+1] = sim_position[i] + sim_speed[i] * dt | These lines are used to move the calculations to the next time step. |
| print("Simulation complete!") | This prints a string to tell us the simulation is complete |
| plt.figure(figsize = (12,5)) plt.plot(time_data, leader_position, 'k-', linewidth = 2, label = 'Leader (NGSIM)') plt.plot(time_data, data_subset['follower_position(m)'].values, 'y-', linewidth = 2, label = 'Real Follower (NGSIM)') plt.plot(time_data, sim_position, 'm--', linewidth = 2, label = 'Simulated Follower (IDM)') plt.xlabel('Time (seconds)') plt.ylabel('Position (m)') plt.legend() plt.grid(True, alpha = 0.5) plt.show() plt.savefig("Simulation Study.png", dpi = 300, bbox_inches = "tight") | These last lines of code are used to put the calculations into a plot to easily visualize the results. |